Isle of Wight catchment

River Basin Management Plan

Cycle 2, 2015-21

- What Isle of Wight waterbodies are failing for according to the Water Framework Directive

- Objectives for cycle 2 of RBMP (2015-21)

- Likely action to address the fail

WFD classifications for surface water, are a pass: High or Good, or three levels of fail: Moderate; Poor;  Bad. Groundwater is rated as Good or Poor.

Objectives are to address all the issues listed for each waterbody by the date specified. The target is to get to Good Ecological Status GES (if the waterbody is not modified) or Good Ecological Potential GEP (if modified).

Blackbridge Brook GB107101006100

Overall status Moderate for this heavily modified waterbody. This is the upstream waterbody of Wootton Creek transitional and coastal waterbody.

Objective to be Good by 2021 is optimistic.

| Issues | Actions needed |
| --- | --- |
| Phosphate moderate Diffuse pollution is having a negative impact on the water quality, contributing to the failure. Waterbody wasn’t classified for phosphate in 2009, but now has new data based on one monitoring site, G0006279 Havenstreet. The failure has not been attributed to any STW. Modelling has been undertaken and this indicates that 90% of the phosphate in the waterbody is diffuse pollution related. Farm visits to be undertaken to raise awareness and resolve any issues identified. Other sources to consider railway station and fishing lake. | Stage 1 investigation, we need to try and pinpoint where the issues are in this large catchment. Sampling to be undertaken above roadbrige at Havenstreet. More specific actions may come out of this. |
| Invertebrates are Moderate. |  |
| Macrophytes and Phytobenthos Combined (Phytobenthos) are Moderate. Related to phosphate failures diffuse pollution and natural low flows. |  |
| Mitigation Measures are needed for channel modifications, and is Moderate. | Actions identified by APT from walkover including: modify weir to aid fish passage, thin or remove deciduous plantation and restore geomorphological features. This would also benefit Briddlesford CopsesValue (SSSI). A project “Blackbridge Brook Benefits” has been proposed to deliver these. There has been consideration of the removal of the weir, making the lake above weir tidal. Possibly Island Roads doing work there. Would need discussions with NE (protected worm) and Hotel owners. |